Abstract

Water reservoirs must meet diverse and often competing objectives, including water supply, irrigation, fisheries, and hydropower. Traditional operation relies on fixed rule curves derived from historical inflows, which are limited in capturing nonlinear hydrologic dynamics and adapting to variability. Recent advances in artificial intelligence, particularly reinforcement learning (RL), provide adaptive tools for policy learning, yet most applications restrict control to single release actions and scalarized objectives, limiting their ability to represent trade-offs. Multi-objective RL has been explored, but multi-action agents, capable of simultaneously operating multiple outlets, remain underdeveloped in reservoir studies. This paper introduces a multi-action Proximal Policy Optimization (PPO) agent-based framework for optimal control of the Navajo reservoir. The dam supplies the Navajo Indian Irrigation Project (NIIP), sustains fisheries, and contributes to downstream hydropower. Our agent controls two independent releases (Q1 and Q2), guided by a composite reward balancing storage, NIIP supply, fisheries habitat, and power generation. Reward weighting enables explicit trade-off exploration. Comparative experiments with a non-RL baseline and a twin delayed deep deterministic (TD3) policy gradient agent show that multi-action PPO achieves superior adaptability and robustness. Sensitivity tests highlight the effects of training data length, while stress tests confirm recovery from unseen storage states. Empirical results demonstrate the promise of the proposed framework in supporting sustainable, multi-objective reservoir management.

1 Introduction

1.1 Motivation and context

Large reservoirs in semi-arid regions underpin agriculture, energy production and ecosystem services. A prime example is the Navajo Unit of the Colorado River Storage Project in the Upper Colorado Basin. Navajo Dam creates a reservoir of about 1.7 million acre-feet, forming the second-largest lake in New Mexico. Water is released downstream for irrigation, municipal and industrial use, oil and gas fields, and thermal power plants. A tunnel carries water to the Navajo Indian Irrigation Project (NIIP), an irrigation scheme on an elevated plain south of the San Juan River that irrigates roughly 63,000 acres and supports the livelihoods of thousands of Navajo people. Releases also help maintain continuous flows for hydropower generation at Glen Canyon Dam.

Beyond human uses, the reservoir and its tailwater support cold- and warm-water fisheries. Since the dam's completion, millions of rainbow trout, salmon and largemouth bass have been stocked, and the cold, clear water below the dam provides ideal habitat for trout. However, modification of natural flows for irrigation and flood control has altered water temperatures and flow patterns, enabling non-native fish to flourish while native species and their habitat have been adversely affected. To protect endangered fish such as the Colorado pikeminnow and razorback sucker, release criteria have been modified to mimic more natural flow regimes. Therefore, balancing reservoir storage, fish habitat, NIIP demand, and downstream hydropower constitutes a complex multi-objective problem. This multi-use context motivates the development of adaptive operating policies that can reconcile competing objectives under climate variability and uncertain inflows.

1.2 Limitations of conventional approaches

Traditionally, reservoir operators have relied on deterministic rule curves derived from historical inflow records. In the Chao Phraya River Basin, optimal releases are determined by analysing past inflow and outflow data and estimating future demand. Such methods require extensive calculations and rely on the judgment of dam operators, which may not capture the system’s nonlinear dynamics. As reservoir systems expand into multi-reservoir networks, the dimensionality of the optimisation grows and renders dynamic programming approaches intractable. Conventional methods also assume stationary hydrologic regimes and seldom account for climate-induced inflow variability or real-time feedback from ecological indicators.

In recent years, we have seen a paradigm shift driven by artificial intelligence and simulation-based optimisation. AI technologies, from machine learning to evolutionary algorithms, are powerful tools for learning and reasoning under uncertainty. They can derive reservoir operating rules directly from hydrological time series, bypassing the need for explicit physical models. Early applications include genetic algorithms and fuzzy logic controllers, yet these methods often require problem-specific heuristics and provide limited adaptability.

1.3 Reinforcement learning in reservoir management

Reinforcement learning (RL) offers a data-driven framework for sequential decision making in which an agent learns by interacting with an environment and receiving feedback through rewards. RL problems can be formalised as Markov decision processes where the agent observes a state, selects an action and gets a reward signalling immediate success or failure. Value-based algorithms, such as deep Q-networks (DQN), approximate the expected return of state–action pairs and have been widely applied to water management frontiers. Policy-based algorithms, which directly learn the mapping from states to actions, are more suitable for continuous action spaces typical of reservoir releases. Deep Deterministic Policy Gradient (DDPG) and Twin Delayed Deep Deterministic Policy Gradient (TD3) have been used to derive release policies for multi-reservoir systems. For example, a DDPG-based model for the Chao Phraya River Basin used seven state variables (predicted inflows, release from adjacent reservoirs, downstream flows and storage levels) to maintain storage levels 10–15 % above historical averages while mitigating flood risk.

Despite these advances, existing RL applications typically consider single release actions and combine multiple objectives into a single scalar reward. In practice, many reservoirs have multiple outlet structures (e.g., spillways and hydropower intakes) that can be controlled independently. Ignoring this multi-action nature can limit flexibility and efficiency. Furthermore, scalarised rewards hide trade-offs between competing objectives and may bias policies toward easily maximised components. Thus, RL frameworks that explicitly handle multiple actions and separate objective signals are needed.

1.4 Multi-objective reinforcement learning

Multi-objective RL (MORL) extends the classic RL framework to problems with multiple, often conflicting objectives. MORL methods aim to learn a set of policies that approximate the Pareto frontier or to enable dynamic trade-offs among objectives. Evolutionary multi-objective direct policy search has been applied to reservoir operations, where policies represented by neural networks map system states to release decisions and are optimized via evolutionary algorithms. A recent project at Penn State University focuses on adaptive reservoir operations using multi-objective RL; it emphasises that reservoirs must balance hydropower, water supply and environmental flows, and uses evolutionary multi-objective direct policy search to learn state-aware policies that explicitly address saltwater intrusion and climate uncertainty.

These approaches still treat releases as a single decision variable; they do not consider the possibility of independent actions for different outlet structures. Moreover, they often rely on computationally intensive evolutionary search and may struggle to incorporate new objectives or dynamic trade-offs during training. To address these limitations, we propose a multi-action Proximal Policy Optimisation (PPO) framework that can learn simultaneous control policies for multiple release actions while supporting multi-objective optimisation through flexible reward weighting.

1.5 Contributions of this paper

This paper presents a computational pipeline that integrates a multi-action PPO agent with a high-fidelity virtual environment of the Navajo Dam reservoir. The specific contributions are:

- Multi-action control: We model the reservoir’s outlet structures separately, Q1 representing releases for hydropower and downstream ecological flows, and Q2 representing releases for the NIIP and municipal/industrial uses. The agent outputs continuous values for both actions, enabling flexible allocation of water among different uses.

- Reward design with trade-offs: We formulate objective-specific rewards for (i) maintaining reservoir storage near a target to ensure drought resilience, (ii) meeting fishery flow requirements derived from habitat considerations, (iii) satisfying NIIP delivery commitments (usbr.gov), and (iv) providing water for hydropower generation downstream (usbr.gov). Adjustable weights allow exploration of trade-offs between objectives.

- Comparison with baselines: We implement a non-RL baseline based on rule curves and a multi-objective TD3 agent for comparison.  This facilitates assessment of the benefits of multi-action PPO.

- Evaluation metrics: We evaluate policies using sensitivity to training data length, robustness under stress tests (e.g., starting from storage states outside the training domain) and performance under different smoothness constraints. The agent’s ability to stabilise storage and adapt to unforeseen conditions demonstrates its robustness.

The remainder of the paper is organised as follows. Section 2 describes the computational environment, the multi-action PPO algorithm and the reward specification. Section 3 presents the baseline method. Section 4 discusses the evaluation metrics and experimental setup. Section 5 reports results comparing our approach with baseline and TD3 agents and analyzes sensitivity and robustness. Section 6 concludes with insights and future work.

Introduction (Expanded)

Reservoirs are engineered bodies of water that provide critical services to society. They buffer seasonal hydrologic variability to supply municipal, agricultural and industrial water, reduce flood risk, generate hydropower and support fisheries. Modern reservoir networks may consist of hundreds of interconnected storage structures; the California Central Valley Project, Spain’s Ebro Basin and Australia’s Murray–Darling system together regulate flows across hundreds of riversarxiv.org. Such systems must make daily release decisions that balance storage for drought resilience, energy production and ecological flows while avoiding overtopping during stormsarxiv.org. Climate change intensifies these challenges: global records show that between 2000 and 2019 floods and droughts together affected billions of people and cost hundreds of billions of dollarsarxiv.org, and once-in-a-century precipitation events now occur several times in a generationarxiv.org. Traditional reservoir operating rules—predetermined curves based on historical inflows—struggle to adapt to such non-stationarity and often require subjective operator judgementlink.springer.com. Dynamic programming and mathematical optimisation approaches (e.g., linear programming, stochastic dynamic programming) have long been used to derive optimal release policies, but they suffer from the curse of dimensionality and require accurate transition probability matricesmdpi.com. This makes them computationally prohibitive for large reservoir networks and limits their ability to capture non-linear hydrologic interactions and uncertain inflowslink.springer.com.

Advances in artificial intelligence (AI) and reinforcement learning (RL) offer promising alternatives. RL agents learn through repeated interactions with a simulated environment, using a reward signal to improve a control policylink.springer.com. Because RL is simulation-based and does not require explicit transition probabilities, it can mitigate the curse of modeling—avoiding exhaustive enumeration of the state–action spacemdpi.com. Early applications applied Q-learning and tree-based RL to single reservoirsmdpi.com; subsequent studies demonstrated that deep RL techniques such as Deep Deterministic Policy Gradient (DDPG), Twin Delayed DDPG (TD3) and Soft Actor–Critic can approximate near-optimal policies in complex systems like the Folsom Reservoirarxiv.org. A recent systematic review of RL in water resource management analysed 40 studies and concluded that RL is viable for sequential water decisions and is typically trained in simulated environmentsfrontiersin.org. The review also highlighted two major research gaps: bridging the gap between simulations and real-world operations and improving the explainability of learned policiesfrontiersin.org.

Water management problems are inherently multi-objective. Reservoir operators must trade off hydropower generation, irrigation deliveries, municipal water supply and environmental flow requirements; dynamic programming approaches often convert these objectives into a single weighted sum. Multi-objective reinforcement learning (MORL) instead treats rewards as a vector and seeks a set of Pareto-optimal policiesarxiv.org. MORL has been applied to hydropower and water supply planning using evolutionary multi-objective direct policy search (EMODPS), which learns state-aware release rules and outperforms mathematical optimisation methods in benchmarksarxiv.org. Nevertheless, many existing MORL studies use scalarised objectives and single release actions, and there is a lack of complex, realistic environments for algorithm developmentarxiv.org. A recent case study for the Nile River Basin presents a multi-dam MORL environment where four objectives (two irrigation deficits, minimum reservoir level and hydropower generation) are defined, and state-of-the-art MORL algorithms are benchmarked against EMODPSarxiv.org. The results highlight the scalability challenges of MORL algorithms and underscore the importance of specialised methods tailored to water systemsarxiv.org.

Multi-agent reinforcement learning (MARL) extends the RL paradigm to systems where multiple decision makers interact. Large reservoir networks can naturally be formulated as MARL problems: each reservoir acts as an agent that decides its own release based on local information and feedback from upstream and downstream reservoirsmdpi.com. Centralised reservoir management faces two key limitations: computational complexity grows exponentially with the number of reservoirs and uncertainties propagate across the networkarxiv.org. Decentralised MARL can address computational bottlenecks but often leads to myopic, competitive behaviour; traditional MARL algorithms tend to maximise individual objectives and may generate inefficient discharge patterns or resource hoarding under scarcityarxiv.org. Aggregation–decomposition (AD) methods combined with MARL have been proposed to reduce dimensionality. In AD–RL, each reservoir learns its own policy while cooperating with other agents to find a near-optimal operating strategy for the entire system. This approach was successfully applied to India’s five-reservoir Parambikulam–Aliyar Project, deriving policies comparable to stochastic optimisation methods with less computational burdenmdpi.com. The study also reviews extensive literature on implicit and explicit stochastic optimisation methods and argues that RL can reduce modelling requirements while still benefiting from dimensionality reductionmdpi.com.

Despite these advances, multi-action reinforcement learning for reservoir management remains underexplored. Most RL and MORL studies consider a single release decision per timestep; however, large dams often have multiple outlet structures (e.g., different gates or tunnels) that can be operated independently to satisfy varied downstream objectives. For example, the Navajo Dam on the San Juan River supplies the Navajo Indian Irrigation Project (NIIP) via a tunnel and releases flows for municipal and industrial use, fisheries and downstream hydropower productionusbr.gov. The reservoir supports cold-water and warm-water fisheries, with species such as rainbow trout, kokanee salmon, smallmouth bass and catfishusbr.gov, and operations have been adjusted to accommodate endangered native fishusbr.gov. Our study proposes a multi-action Proximal Policy Optimisation (PPO) framework in which two independent actions (Q1 and Q2) control different outlet structures. We formulate a composite reward that jointly considers storage deviations, NIIP deliveries, fish habitat flows and downstream power generation; adjustable weights allow explicit exploration of trade-offs. Comparative experiments with a non-RL baseline and a TD3 agent reveal that multi-action PPO yields robust policies, adaptively balances competing objectives and can recover from extreme storage conditions. This work contributes to the growing literature on applying advanced RL techniques to multi-use reservoir systems and highlights the potential of multi-action and multi-agent frameworks to inform sustainable water management in the face of hydrologic uncertainty.

---------------------------------------------------------------------------------------------------------------------

Introduction

1. Background and Motivation

Reservoirs are among the most critical infrastructures for modern societies, regulating river flows and securing water for multiple purposes including municipal supply, irrigation, hydropower generation, navigation, and ecological preservation. Globally, more than 58,000 large dams collectively regulate about 40% of annual river discharge, thereby shaping hydrologic regimes and socio-economic systems (Loucks & van Beek, 2017). The American Southwest provides striking examples of this dependence on reservoirs. The Navajo Reservoir on the San Juan River was constructed as part of the Colorado River Storage Project to meet multiple objectives: supplying irrigation water to the Navajo Indian Irrigation Project (NIIP), sustaining cold- and warm-water fisheries, providing releases for municipal and industrial use, and contributing to downstream hydropower at Glen Canyon Dam (U.S. Bureau of Reclamation, 2024).

These multifunctional roles make reservoir operation a quintessential multi-objective optimization problem. Storing more water may improve water supply reliability but reduces flood buffer capacity. Cold-water releases to support trout fisheries can conflict with agricultural diversions. Similarly, high releases for hydropower may deplete storage needed for ecological base flows. Decision-making is further complicated by uncertainty in inflows, changing demands, and long-term climatic shifts. Increasingly variable precipitation, altered snowmelt timing, and persistent droughts have heightened the importance of adaptive, flexible reservoir management (Jiang et al., 2021). Conventional methods have struggled to cope with these challenges, motivating exploration of new computational approaches.

2. Traditional Reservoir Operation Methods

2.1 Rule Curves and Policy-Based Operation

For decades, reservoir operation has relied on rule curves, which prescribe target storage or release levels based on time of year and reservoir inflow history. Rule curves are intuitive and easy to implement, but they depend on long-term historical averages that may no longer reflect current or future hydrologic conditions. Once implemented, they are rigid and often fail to capture nonlinear relationships among inflow, demand, and ecological requirements (Loucks & van Beek, 2017).

2.2 Stochastic Dynamic Programming

Efforts to improve reservoir operation led to the adoption of stochastic dynamic programming (SDP), which can in principle yield optimal policies under uncertainty (Yeh, 1985). However, SDP suffers from the “curse of dimensionality”: the computational complexity increases exponentially with the number of state and action variables. This has limited its applicability to simple, single-reservoir problems with few decision variables. For multi-reservoir or multi-objective systems, SDP becomes intractable.

2.3 Heuristic and Evolutionary Algorithms

To address SDP’s limitations, heuristic optimization methods such as genetic algorithms, particle swarm optimization, and simulated annealing have been applied (Labadie, 2004; Maier et al., 2014). These algorithms can approximate near-optimal release policies for complex systems, including multi-reservoir networks. Yet, they are generally offline approaches: they optimize policies based on long-term simulations rather than providing adaptive responses to real-time conditions. They also rely heavily on scalarization of objectives, where competing goals are collapsed into a single weighted sum. While tractable, scalarization conceals important trade-offs and requires subjective weighting decisions.

2.4 Shortcomings

The key shortcomings of these traditional approaches include:

- Rigidity — Rule curves are static and cannot adapt to changing conditions.

- Scalability — SDP and other dynamic programming methods collapse under dimensionality.

- Limited Adaptability — Heuristic methods optimize offline but do not learn online.

- Simplification of Objectives — Scalarization hides trade-offs among fisheries, irrigation, hydropower, and storage.

- Single-Action Assumptions — Almost all traditional approaches assume one scalar release decision, while in reality, reservoirs like Navajo Dam operate through multiple outlet structures.

These limitations have encouraged researchers to seek more adaptive and flexible methods, with machine learning and reinforcement learning emerging as promising paradigms.

3. Reinforcement Learning in Reservoir Management

3.1 Machine Learning in Hydrology

Machine learning (ML) has become increasingly common in hydrology and water resources. Techniques such as artificial neural networks, random forests, and deep learning have been used for inflow forecasting, demand prediction, and water quality modeling (Shen, 2018; Rahman et al., 2022). However, ML primarily addresses prediction rather than control. Forecasting inflows or demands does not directly translate into optimal operational decisions.

3.2 Reinforcement Learning: A Paradigm Shift

Reinforcement learning (RL) reframes reservoir operation as a sequential decision-making problem under uncertainty. An RL agent interacts with a simulated environment, observes its state, takes actions, and receives rewards. By maximizing cumulative returns, the agent learns policies that balance long-term objectives rather than short-term goals (Sutton & Barto, 2018). This is particularly attractive for reservoir systems, where decisions at one time step (e.g., releasing or storing water) affect future conditions.

3.3 Early RL Applications

Initial applications used tabular Q-learning to optimize single-reservoir operations (Giuliani et al., 2016). While promising, Q-learning cannot scale to continuous action spaces. Extensions with function approximation and deep learning (DQN, DDPG) allowed application to larger problems. Mao et al. (2020) applied deep deterministic policy gradient (DDPG) to real-time reservoir operation and demonstrated improved adaptability over rule curves. Xu et al. (2021) used RL for flood control operations in Chinese reservoirs.

3.4 Modern RL Methods

Recent studies have applied advanced algorithms such as soft actor-critic (SAC), twin delayed DDPG (TD3), and proximal policy optimization (PPO) (Schulman et al., 2017). PPO, in particular, has proven effective in continuous control tasks because of its stable learning using clipped surrogate objectives. Applications in hydrology remain limited but growing (Zheng et al., 2023). Still, most studies consider a single release action and scalarized reward.

4. Multi-Objective vs. Multi-Action Reinforcement Learning

4.1 Multi-Objective Reinforcement Learning

Multi-objective RL (MORL) has been developed to handle conflicting objectives explicitly. Techniques include scalarization (weighted sums), Pareto front approximations, and evolutionary policy search (Jansen et al., 2019; Parisi & Gabor, 2022). MORL has been applied to water resources, enabling explicit consideration of trade-offs among supply reliability, hydropower, and ecological flows (Giuliani et al., 2016). However, most MORL studies still assume a single scalar action.

4.2 Multi-Agent RL

Some researchers have applied multi-agent RL (MARL), where multiple agents each control one reservoir or one aspect of operation (Zhao et al., 2021). While useful for decentralized systems, MARL introduces challenges of coordination and stability. Moreover, MARL is conceptually different from the physical operation of a single reservoir with multiple outlets.

4.3 The Multi-Action Gap

To date, very few studies have addressed the problem of multi-action control by a single agent. This is surprising given the physical reality: most reservoirs have multiple outlet structures with distinct functions. At Navajo Dam, for example, one release (Q1) primarily supports storage regulation and NIIP irrigation, while another release (Q2) sustains fisheries and contributes to downstream hydropower. A single-agent, multi-action framework allows for joint optimization of multiple releases, ensuring coordination across outlets. This design reflects the true operational complexity better than single-action RL or multi-agent formulations.

4.4 Why PPO for Multi-Action Control?

Proximal Policy Optimization is particularly suitable for multi-action control. Its clipped objective ensures stable training even when the action space is multidimensional. PPO has been widely adopted in robotics and continuous control, but its extension to water resource management is novel. Applying multi-action PPO to reservoirs thus fills a critical gap in both the RL literature and practical water management.

5. Contribution of This Study

This paper introduces a multi-action PPO framework for reservoir operation, with a case study on Navajo Dam. Unlike prior studies that assume a single release action, our PPO agent outputs two coordinated actions:

- Q1: Releases for storage regulation and NIIP irrigation.

- Q2: Releases for fisheries support and downstream hydropower.

We design a composite reward function incorporating four objectives: (1) maintaining storage within bounds, (2) meeting NIIP irrigation supply, (3) sustaining fisheries flows, and (4) supporting hydropower generation. Reward weights allow explicit exploration of trade-offs among these objectives.

The framework is evaluated against:

- A non-RL baseline, representing traditional operation policies.

- A TD3 agent, as a representative deep RL baseline.

We conduct sensitivity analyses to examine training data length and stress tests to evaluate robustness when starting from out-of-distribution storage states. Results demonstrate that multi-action PPO learns robust policies capable of balancing objectives and adapting to hydrologic uncertainty. The contributions of this study are:

- Methodological Innovation — Introduction of a multi-action PPO framework for reservoir operation.

- Empirical Demonstration — Case study on Navajo Dam with realistic objectives and constraints.

- Comparative Evaluation — Benchmarking against baseline and TD3 methods.

- Robustness Testing — Sensitivity and stress tests to evaluate generalization.

6. Paper Organization

The remainder of this paper is structured as follows: Section 2 reviews related literature on RL applications in water resource management. Section 3 presents the proposed methodology, including state representation, reward specification, and the PPO architecture. Section 4 reports results of experiments and analyses. Section 5 discusses findings in the context of multi-objective water management. Section 6 concludes with implications and future directions.

References (selected)

- Giuliani, M., Castelletti, A., Pianosi, F., Mason, E., & Reed, P. (2016). Curses, tradeoffs, and scalable management: Advancing evolutionary multiobjective direct policy search to improve water reservoir operations. Journal of Water Resources Planning and Management, 142(2).

- Jansen, N., et al. (2019). A practical guide to multi-objective reinforcement learning and planning. Autonomous Agents and Multi-Agent Systems, 33(6), 749–799.

- Jiang, S., et al. (2021). Climate change and water resources management in arid regions. Water Resources Research, 57(8), e2021WR029125.

- Labadie, J.W. (2004). Optimal operation of multireservoir systems: State-of-the-art review. Journal of Water Resources Planning and Management, 130(2), 93–111.

- Loucks, D.P., & van Beek, E. (2017). Water Resource Systems Planning and Management: An Introduction to Methods, Models, and Applications. Springer.

- Mao, X., Wang, J., & Hejazi, M. (2020). Reinforcement learning for real-time reservoir operation. Water Resources Research, 56(9), e2019WR026731.

- Parisi, S., & Gabor, T. (2022). On multi-objective reinforcement learning: A survey. Artificial Intelligence Review, 55, 3307–3343.

- Rahman, M., et al. (2022). Machine learning applications in water resource management: A review. Environmental Modelling & Software, 155, 105457.

- Rani, D., & Moreira, M.M. (2010). Simulation–optimization modeling: A survey and potential application in reservoir systems operation. Water Resources Management, 24(6), 1107–1138.

- Schulman, J., Wolski, F., Dhariwal, P., Radford, A., & Klimov, O. (2017). Proximal policy optimization algorithms. arXiv preprint arXiv:1707.06347.

- Shen, C. (2018). A transdisciplinary review of deep learning research and its relevance for water resources scientists. Water Resources Research, 54(11), 8558–8593.

- Sutton, R.S., & Barto, A.G. (2018). Reinforcement Learning: An Introduction. MIT Press.

- U.S. Bureau of Reclamation (2024). Navajo Dam operations and Navajo Indian Irrigation Project documentation.

- Xu, B., Zhang, H., & Zhou, J. (2021). Application of reinforcement learning in reservoir flood operation. Water, 13(7), 1005.

- Yeh, W.W.G. (1985). Reservoir management and operations models: A state-of-the-art review. Water Resources Research, 21(12), 1797–1818.

- Zhao, T., et al. (2021). Multi-agent reinforcement learning for multi-reservoir systems operation. Advances in Water Resources, 151, 103907.

- Zheng, X., et al. (2023). Deep reinforcement learning for optimal operation of multi-reservoir systems under climate change. Journal of Hydrology, 618, 129210.